- Description

This work shall be done in accordance with sections 201 of the Michigan Department of Transportation 2020 Standard Specifications for Construction, except as herein provided.

- Materials

All work must be in accordance with the contract documents.

- Construction

- Removals

Remove trees 3-inches in diameter and greater, within 18-inches of existing ground. Existing stumps, or stumps generated from tree removals will be removed with a future contract. The removal limits shall extend to 3 feet beyond the proposed grading limit line (slope stake line) or to the right-of-way line, whichever is less, and including other areas designated on the plans. Grading limits include vertical curves at the top of cut sections; toe of fill slopes and as shown on the plans. Trees shall be felled without danger to traffic or injury to other trees, structures or property. Trees are not to be removed until verified by the Engineer.

- Measurement and Payment

The completed work as measured includes all labor and equipment will be paid for at the contract unit price for the following pay item:

Pay Item	Pay Unit

Clearing, Modified, RCOC	Acre

Clearing, Modified, RCOC	Syd