# GENERAL NOTES

# NOTIFICATION FOR LAYOUT AND INSPECTION

Contractors must notify the RCOC Traffic Operations Engineer by submitting the Contractors-Sign/Striping Inspection Form via the web form at https://www.rcocweb.org/. Navigate to Doing Business with RCOC and select the form.

Inspection of work must be requested by 3:00 PM the day prior to when work is to occur as described in section A. Inspection for Saturday, Sunday and Monday must be scheduled by 3:00 PM on the preceding Friday.

Failing to provide proper notification as described above may be denied inspection due to unavailability of staff.

Sign and pavement markings placed by Contractor without RCOC Traffic Operations Engineer’s approval will not be paid for.

# LAYOUT RESPONSIBILITY AND ADVANCE NOTIFICATION

## TEMPORARY CONSTRUCTION SIGNING AND PAVEMENT MARKINGS

Contractors must mark construction sign locations in the field. This work is included in the project cost.  Provide at least one week's notice for field reviews before installation. Unapproved signs must be reinstalled at the contractor's expense if deemed improperly placed.

Contractors must provide at least one week's notice for RCOC layout before installation.  RCOC Traffic Operations Engineer will mark temporary pavement markings.

## PERMANENT SIGNING AND PAVEMENT MARKINGS

Contractors must provide at least one week's notice for layout before markings are installed. The RCOC Traffic Operations Engineer will layout permanent signing and pavement markings.

# TEMPORARY CONSTRUCTION SIGNING AND PAVEMENT MARKINGS

## Speed Limit

Posted Speed Limit (before work zone): XX MPH

Work Zone Speed Limit: XX MPH

## Portable Changeable Message Signs (PCMS’s)

PCMS must display pre-approved messages 72 hours after project award or at least one week before project commencement, as approved by the RCOC Traffic Operations Engineer.

## Construction Project Special Signs

Install signs indicating the project or road closure start date promptly after award or two weeks prior to construction. No work begins until these signs are in place.

## Sign Supports, and Barricades

Temporary supports for construction signs should not exceed 14 days of use. For longer durations, use ground-mounted U-channel posts.

Secure Type III barricades with at least eight sandbags (35 lbs. each). Place signs on separate supports behind and above barricades.

At hard closure areas (where vehicles are not to pass) place enough Type III Barricades from curb to curb with no gaps.

Barricades with R11-4 (Road Closed to Thru Traffic) signs are to be placed on shoulder of roadways and not in driving lane.

## Detour Signs

- Name Plaques shall always be mounted above detour signs and below diamond signs

- Signs are placed on both sides of the road for one-way roads and boulevards.

## Blips

If the road is open to traffic, temporary traffic markings are required. The contract includes Type R Tape for blips.

Blips must be at least 4 feet long and spaced every 50 feet on straight sections. Around curves, blips must be 2 feet long and spaced every 25 feet, matching the barrel spacing.

## Sign Covering

- Cover conflicting signs, such as speed limits, lane usage, and warning signs.

## Sign Spacing and Locations

- Install signs as shown, spaced approximately 350 feet apart. Adjust locations as needed for clear sight lines, considering constraints like trees, existing signs, driveways, sidewalks, and shoulders. Spacing between existing signs and installed signs shall be a minimum of 100 feet, or a distance to ensure proper sight lines.

- Place the 'End Road Work' sign as closely and directly across from the 'Work Zone Begins' sign as possible during installation.

## Channelizing Devices

Space channelizing devices 25 feet apart along tapers and 50 feet apart along tangents, or as directed by the engineer. Use tighter spacing in critical areas, such as near driveways and intersections.

If included in the contract, use the provided limited number of barrels (plastic drums) for tapers only, or based on the contractor's or engineer’s direction.

## Pavement Markings

Install and remove temporary pavement markings as directed. Remove conflicting markings before installing new ones.

For work activities lasting seven days or less, tape is optional in specified areas as directed by the Engineer.

# PERMANENT SIGNING AND PAVEMENT MARKING

## Street Name Signs

Street name signs (D3-1) require two blades installed back-to-back on 4-foot square tubes inside U-channels, with a bottom height of 10 feet.  Therefore, two of each sign must be fabricated.

## Permanent Sign Locations

RCOC will mark permanent sign locations in the field with numbers. Contractors will receive corresponding numbered plans at the time of installation.

# PEDESTRIAN TRAFFIC CONTROL

## Pedestrian Detours

## Maintain pedestrian traffic at all times by detouring at intersections, following MMUTCD TA-29 standards.

## Sidewalk Ramp Closures

## Sequence ramp closures to ensure pedestrians can cross intersections throughout construction.

REVISION: 12/09/2024